How to use NEXTA to visualize the trajectory

NGSIM data set

Version 1: Prepared by Xuesong Zhou and Pan Shang

# About Next Generation Simulation (NGSIM)

https://ops.fhwa.dot.gov/trafficanalysistools/ngsim.htm

| Category | File | Remark |
| --- | --- | --- |
| Main program | NEXTA_4_Trajectory.exe | Open source NEXTA visualization tool |
| Input | trajectories-0400-0415 | Sample file from NGSIM |
| Input | test_one_line test_two_lines | Sample file from Pan Shang, for one vehicle’s trajectory |
| Output | Trajectory.csv | Export to more user readable format |

Format

Each line is a record of trajectory

Please use space or comma to separate fields.

| No | Field Name | Example 1 | Example 2 | Unit | remark |
| --- | --- | --- | --- | --- | --- |
| 1 | Vehicle_ID | 1 | 1 |  |  |
| 2 | Frame_ID | 12 | 13 | 0.1 seconds |  |
| 3 | Total_Frames | 884 | 884 | 0.1 seconds |  |
| 4 | Global_Time | 1113433136100 | 1113433136200 | text | you can put any value |
| 5 | Local_X | 16.884 | 16.938 |  | not used |
| 6 | Local_Y | 48.213 | 49.463 | feet |  |
| 7 | Global_X | 6042842.116 | 6042842.012 |  | not used |
| 8 | Global_Y | 2133117.662 | 2133118.909 |  | not used |
| 9 | Vehicle_Length | 14.3 | 14.3 | feet |  |
| 10 | Vehicle_Width | 6.4 | 6.4 | feet |  |
| 11 | Vehicle_Class | 2 | 2 |  | pax or truck |
| 12 | Vehicle_Velocity | 12.5 | 12.5 |  |  |
| 13 | Vehicle_Acceleration | 0 | 0 |  |  |
| 14 | Lane_Identification | 2 | 2 |  | very important |
| 15 | Preceding_Vehicle | 0 | 0 |  |  |
| 16 | Following_Vehicle | 0 | 0 |  |  |
| 17 | Spacing | 0 | 0 |  |  |
| 18 | Headway | 0 | 0 |  |  |

Steps

Step 1: Open NEXTA.

Step 2: Select menu Tools-NGSIM Tools-Active Space-Time View, and then load trajectory file for Visualizing trajectory data

Step 3:

Main functions

- Display trajectories for each lane.

In the menu Tools-NASIM Tools:

You can choose each lane number to see the trajectory;

- You can show Vehicle ID, and you can use mouse wheeler to zoom in and zoom out the display in the certain portion.

- You can use the mouse to click and draw a line in the space-time plate, and the NeXTA will automatically calculate the flow, density, and speed values across the line

- You can show cumulative flow count and density

You can show the space-time Density Contour